Southern California Bight 2023 Regional Monitoring Program: Submerged Aquatic Vegetation (SAV)

SCCWRP Technical Report

Authors:

Southern California Coastal Water Research Project, Costa Mesa, CA

March 2023

BIGHT ‘23 SAV COMMITTEE

| Member | Affiliation |
| --- | --- |

FOREWORD

Citation

ACKNOWLEDGEMENTS

EXECUTIVE SUMMARY

Background

Study Objectives and Approach

Study Findings

Recommendations

TABLE OF CONTENTS

Bight ‘23 SAV Committee

Foreword

Acknowledgements

Executive Summary

Background

Study Objectives and Approach

Study Findings

Recommendations

Table of Contents

List of Figures

List of Tables

- Introduction

- Methods

  - Study Design

  - Field Collection of Structural Metrics

  - Laboratory Measurement of Structural Metrics

  - Index development

  - Additional Analyses

- Quality Assurance and Quality Control

  - Field

  - Laboratory

  - Data analysis

- Results

- Discussion

- Conclusions

- Recommendation

- Literature Cites

Appendix A – Eelgrass structural metrics

Appendix B – Eelgrass index and function scores by waterbody

Appendix C – Maps of eelgrass condition

Appendix D – Exploratory Analyses

List of Figures

List of Tables

- Introduction

- SAV overall ecological importance

- SAV in SCB habitats

- Structure to function relationship

- How SAV fits in with regional monitoring + need for condition tool

- Methods

  - Study Design

A stratified probabilistic sampling approach was used to sample eelgrass beds throughout the Southern California Bight such that inferences may be drawn from the sampled population of eelgrass to the true population of eelgrass. A base map layer from Point Conception, CA to the US-Mexico border was generated from previous surveys of eelgrass beds across the region from 2015-2022 (Merkel and Associates, 2016; Merkel and Associates 2023). This base map was augmented with waterbody-by-waterbody site visits focusing on smaller, heretofore under sampled embayments in the northern parts of the Southern California Bight (e.g., Hollister Ranch lagoon, Gaviota estuary, Refugio estuary) looking for eelgrass in the summer of 2022. These small embayments were surveyed for the presence of eelgrass by paddling and snorkeling (when practical) their waters, recording the presence and type of any SAV that was observed. These observations were then digitized into polygons for inclusion into the base map. Lastly, the base map was amended following consultations with local seagrass experts across the region. All of these eelgrass extent and presence data were combined into a single GIS data layer from which sample sites could subsequently be selected. (https://sccwrp.maps.arcgis.com/apps/webappviewer/index.html?id=fbd66a9bc6174ad7a5cf90b3d5b9405c).

Each waterbody along the Southern California Bight was binned into one of three strata prior to sampling (Gillett et al. 2018), based on environmental setting (Doughty 2019) and local knowledge.

- Large embayments – Enclosed or semi-enclosed waterbodies greater than 8 km2 in area that have a permanent connection to the open ocean (e.g. San Diego Bay, Lower Newport Bay).

- Small embayments – Enclosed or semi-enclosed waterbodies less than 8 km2 in area that have permanent connections to the open ocean (e.g. Newport Back Bay, Marina del Ray).

- Estuaries – Small enclosed waterbodies with a distinct connection to upland freshwater sources and permanently maintained or seasonal/ephemeral connection to the open ocean (Anaheim Bay, Batiquitos Lagoon).

The sample frame of the previously observed eelgrass area in each stratum was divided into 500m2 cells that would represent a potential sample site. Ten sample sites were randomly selected without replacement from each stratum (30 in total) using the sample_strat function in the R package spSurvey (v5.5.1) (Dumel et al 2023). Given the ephemeral nature of eelgrass aggregations that expand and contract in extent from year to year, 30 additional sites were identified for each stratum to serve as replacement sites (i.e., overdraw sites) in case any of the initial 10 sample sites did not have any vegetation in 2024. Eleven additional sites of special concern were identified according to management priority, after the probabilistic sample draw. These sites were surveyed and then assigned a stratum based on their respective waterbodies.

Table. List of sites, waterbodies, and site selection method.

| waterbody | masterid | stratum | site selection |
| --- | --- | --- | --- |
| Agua Hedionda Lagoon | 904AHT001 | estuary | probabilistic |
| Batiquitos Lagoon | 904BLT001 | estuary | probabilistic |
| Mission Bay | 906MBECSR | large_embayment | probabilistic |
| Mission Bay | 906MBMCSC | large_embayment | probabilistic |
| Mission Bay | 906MBVCSA | large_embayment | probabilistic |
| Mission Bay | 906MBVINC | large_embayment | probabilistic |
| Mission Bay | 906MBVIYP | large_embayment | probabilistic |
| San Diego Bay | 910ZNJN01 | large_embayment | probabilistic |
| San Diego Bay | 910ZNJN02 | large_embayment | probabilistic |
| San Diego Bay | 912SDBCPB | large_embayment | probabilistic |
| San Diego Bay | 912SDBSLB | large_embayment | probabilistic |
| San Diego Bay | 912SDCBVP | large_embayment | probabilistic |
| San Diego Bay | 912SDNOST | large_embayment | probabilistic |
| San Diego Bay | 912SDSBPO | large_embayment | probabilistic |
| San Diego Bay | 912SDSIWB | large_embayment | probabilistic |
| San Diego Bay | 912SDSSSC | large_embayment | probabilistic |
| San Diego Bay | 912SDT001 | large_embayment | probabilistic |
| San Diego Bay | 912SDT003 | large_embayment | probabilistic |
| Santa Ana River | B23-SAV-001 | estuary | probabilistic |
| Batiquitos Lagoon | B23-SAV-002 | estuary | probabilistic |
| Seal Beach NWR (Case Pond) | B23-SAV-005 | estuary | probabilistic |
| Seal Beach NWR | B23-SAV-006 | estuary | probabilistic |
| Batiquitos Lagoon | B23-SAV-007 | estuary | probabilistic |
| Agua Hedionda Lagoon | B23-SAV-008 | estuary | probabilistic |
| Seal Beach NWR | B23-SAV-009 | estuary | probabilistic |
| Port of Los Angeles | B23-SAV-011 | small_embayment | probabilistic |
| Huntington - Magnolia Marsh2 | B23-SAV-012 | small_embayment | probabilistic |
| Magnolia Marsh | B23-SAV-012 | small_embayment | probabilistic |
| Newport Bay | B23-SAV-014 | small_embayment | probabilistic |
| Port of Los Angeles | B23-SAV-015 | small_embayment | probabilistic |
| Newport Bay | B23-SAV-018 | small_embayment | probabilistic |
| Port of Los Angeles | B23-SAV-020 | small_embayment | probabilistic |
| San Diego South Bay | B23-SAV-022 | large_embayment | probabilistic |
| San Diego Bay | B23-SAV-023 | large_embayment | probabilistic |
| Newport Bay | B23-SAV-025 | large_embayment | probabilistic |
| San Diego Bay | B23-SAV-026 | large_embayment | probabilistic |
| San Diego South Bay | B23-SAV-027 | large_embayment | probabilistic |
| San Diego Bay | B23-SAV-029 | large_embayment | probabilistic |
| San Diego South Bay | B23-SAV-030 | large_embayment | probabilistic |
| Newport Bay | B23-SAV-031 | small_embayment | probabilistic |
| Bolsa Chica | B23-SAV-032 | estuary | probabilistic |
| Batiquitos Lagoon | B23-SAV-035 | estuary | probabilistic |
| San Diego Estuary | B23-SAV-036 | estuary | probabilistic |
| San Dieguito Lagoon | B23-SAV-060 | estuary | probabilistic |
| Magnolia Marsh | B23-SAV-063 | small_embayment | probabilistic |
| Port of Los Angeles | B23-SAV-064 | small_embayment | probabilistic |
| Dana Point Harbor | B23-SAV-066 | small_embayment | probabilistic |
| Mission Bay | B23-SAV-092 | large_embayment | probabilistic |
| Alamitos Bay | B23-SAV-901 | small_embayment | targeted |
| Alamitos Bay | B23-SAV-902 | small_embayment | targeted |
| Alamitos Bay | B23-SAV-903 | small_embayment | targeted |
| Alamitos Bay | B23-SAV-904 | small_embayment | targeted |
| San Diego Bay (Zuniga Jetty) | B23-SAV-905 | large_embayment | targeted |
| Mission Bay | B23-SAV-906 | large_embayment | targeted |
| San Elijo Lagoon | B23-SAV-907 | estuary | targeted |
| Mission Bay | B23-SAV-908 | large_embayment | targeted |
| King Harbor | B23-SAV-909 | small_embayment | targeted |
| Oceanside Harbor | B23-SAV-910 | small_embayment | targeted |
| La Jolla Cove | B23-SAV-991 | NA | targeted |
| Santa Cruz Island | B23-SAV-Santa Cruz Island | NA | targeted |

  - Field Collection of Structural Metrics

Field protocols for sampling are fully detailed in McCune et al. (2020). All in-water measurements and collection of material were conducted by teams of SCUBA divers and any intertidal portions of a location were omitted from sampling. If seagrass was present at a site, the perimeter of the aggregation was measured. Plants were considered to be of the same aggregation if there was less than 5m of bare sediment between plants. Eelgrass was classified into different growth categories:

- Patch – Eelgrass aggregation 3m or less on the longest axis

- Bed – Eelgrass aggregation between 5 and 60m along its longest axis and at least 5m along its cross axis (typically shallowest point to deepest point)

- Fringing Bed – Eelgrass aggregation between 5 and 60m along its longest axis and less than 5m along its cross axis

- Meadow – Eelgrass aggregation larger than a bed

For the bed growth category, a transect (maximum 50m long) was laid along its longest axis (mid transect) and second transect was laid from the shallowest subtidal point to the deepest point (cross transect) that was roughly perpendicular to the mid transect. See Figure 1 for sampling scheme in each of the different growth categories. Starting at the beginning edge of the mid transect, 0.25m2 quadrats were placed every 10m (i.e., a maximum of 5 mid transect quadrats) on the sediment surface. Similarly, quadrats were placed at the shallow and deep ends of the cross transect. At each quadrat, % cover of seagrass was visually estimated, as was % of drifting macroalgae. Shoot density was estimated by counting all of the distinct eelgrass shoots within the quadrat. Flowering shoot density was estimated as the number of shoots within the quadrat with distinct spathes (leaf structures that enclose the flowers) on their upper ends. For sites classified as patches, 1 or 2 0.25m2 transects were haphazardly placed within the patch, depending on the size of the patch. For sites classified as meadows, a 50m X 20m area was identified and transects and quadrats were placed as normal. For sites classified as fringing beds, only a mid transect was used.

Figure 1- Schematic depicting the transect and quadrat sampling scheme for different growth forms of eelgrass. A The idealized Bed, B – The Idealized Patch, C – Th e Idealized Fringing Bed, and D – The Idealized Meadow. Any intertidal portions of given location were omitted from sampling.

At sites classified as beds, eelgrass material was collected for subsequent lab analyses from the first 3 mid transect quadrats and 2 cross transect quadrats. A 10cm i.d. ring was placed in the middle of the quadrat. All above ground material (i.e., shoot base, leaves, and attached epiphytes) within the ring were cut at the sediment surface and placed in sample bags. All below ground material (i.e., roots and rhizomes) within the diameter of the ring were cut, removed from the sediment, gently rinsed, and placed in sample bags. For sites classified as patches, above and below ground material was collected within each quadrat. For sites classified as fringing beds, material was only collected from the first 3 mid transect quadrats. Sites classified as meadows, were sampled the same as bed sites. Further details, including SOPs, can be found in the B23 SAV Workplan document.

  - Laboratory Measurement of Structural Metrics

Within the above ground material from a given site and quadrat, the total number of shoots and the number of leaves per shoot were recorded. All epiphytic material was gently removed from each leaf with a scalpel or glass microscope slide and pooled together for biomass measurement. The length of the of the longest leaf within each shoot was recorded as shoot height and the width of that longest leaf was recorded as shoot width. The total area of all the leaves in each shoot was measured with a leaf area meter (LI-COR™ LI-3100C Area Meter) or a calibrated benchtop scanner and ImageJ image analysis software (https://imagej.net/ij/) and recorded as leaf area. All epiphytic tissues, above ground eelgrass tissue, and below ground eelgrass tissue were blotted dry and measured for wet mass. Each type of tissue was then dried at 60°C for 48 hours and measured for dry mass. A subsample of approximately 2.5 - 3 mg was clipped from each dried tissue sample to measure TOC/TN. Dried samples were subsequently combusted at 500°C for 8 hours and measured for ash free dry mass. The mass of each TOC/TN subsample was multiplied by the ash:dry mass ratio to produce a corrected AFDM for each tissue type, to correct for the removal of tissue. Further details, including SOPs used in this study and proposed for future studies, can be found in the B23 SAV Workplan document.

Table 2 Eelgrass plant and bed-scale metrics measured as part of this protocol and which step details their collection.

| Metric | Field or Lab Data Generation | SOP Step |
| --- | --- | --- |
| Above Ground Biomass | Lab | Step 19 |
| Above Ground C & N | Lab | Step 21 |
| Bed Perimeter | Field | Step 4 |
| Below Ground Biomass | Lab | Step 19 |
| Below Ground C & N | Lab | Step 21 |
| Epiphyte Biomass | Lab | Step 16 |
| Leaf Area | Lab | Step 18 |
| Leaves per Shoot | Lab | Step 17 |
| Percent Cover | Field | Step 7-3 |
| Sediment C & N | Lab | Step 21 |
| Shoot Density | Field and Lab | Steps 7-4 & 14 |
| Shoot Height | Field and Lab | Steps 7-5 & 15 |

  - Index development

Eelgrass condition was quantified using the framework outlined in McCune et al. 2020 and refined using the SCCWRP Regional Eelgrass Survey of Condition and Quality (RESCQ) project data (Table 1). Both Bight and RESCQ Zostera marina structural metrics datasets (89 sites in total) were included in the calibration of the index. Overall condition and individual ecosystem function condition presented here are only for sites as part of the Bight survey. More information regarding the RESCQ survey can be found here (LINK – RESCQ Tech Memo/Final Report).

In brief, individual structural metrics (e.g. shoot height, shoot density, leaf area) were binned into quartiles based their position in the regional dataset (1 = lowest quartile, 4 = highest quartile). These quartile scores were then combined for each of the seven ecosystem function (e.g. primary production, carbon sequestration, substrate stabilization) based on available data and averaged (Table 1). Ecosystem function scores (1-4) were then binned into four categories (Poor, Moderate, Good, Excellent) based on the regional dataset (scores 1.00 to 1.75 = Poor; scores 1.76 to 2.50 = Moderate; scores 2.51 to 3.25 = Good; score 3.26 to 4 = Excellent). All seven ecosystem function scores were averaged together for each site, representing the Overall Condition score, which was binned into the same four condition categories as described above.

  - Additional Analyses

We explored the responsiveness of the eelgrass index based on data availability and Bight committee interest. As a group, the main interest was to evaluate the index’s response to stressors or known “reference conditions”. Data from concurrently taken water samples measured for Total Suspended Solids (TSS) and chlorophyll a was used opportunistically to evaluate index response to potential stressors. Measurements of TSS and chlorophyll a were not available for all sites. Overall, it was hypothesized that if a large enough range in these water quality measurements was observed, the eelgrass index may show a response to chlorophyll and/or TSS as eelgrass stressors.

Additionally, distance from nearest inlet (km) from each site to the threshold of the closest inlet “as the fish swims” was measured using ArcGIS. While we acknowledge that distance from inlet is not a direct measurement of any particular stressor, and is only a cursory approximation of confounding estuarine conditions, it may offer some insight to eelgrass condition. It was hypothesized that eelgrass very far away from the inlet (within an estuary) may experience more stress as a function of its proximity to runoff from the land and higher residence time of polluted water.

Each site was also categorized, based on its location relative to a Marine Protected Area, as either “No Take”, “Limited Take” or “Unprotected”, based on activities permitted in each MPA. A few remaining sites were categorized as “National Wildlife Refuge”.  Site protection categories were determined by overlaying the site coordinates with California Department of Fish and Wildlife’s Interactive MPA Map. Overall, it was hypothesized that the level of site protection may be reflected in the estimated eelgrass condition, either based on protection offering better eelgrass condition, or perceived quality of eelgrass condition influencing management protection decisions.

Relationships between estimated eelgrass condition and the aforementioned response variables were explored using linear regressions, t-tests, and ANOVAs, as appropriate.

- Quality Assurance and Quality Control

The goal of the QA/QC effort explored in this first round of data collection across the Southern California Bight was to ensure the data generated by study participants was comparable and complete. Through quality control practices such as field intercalibration, field and lab resampling, and error tracking, we ensure high data quality for subsequent analyses. Each quality control practice is summarized in the subsections below. Full details can be found in the B23 SAV Workplan.

  - Field

Before data collection in 2023, a field intercalibration was hosted in April 2024 to ensure quality of visual data recorded was consistent across participating laboratories. Labs included CSULB, NOAA Fisheries, Merkel & Associates, LLC, Moss Landing Marine Lab, Paua Marine, and Marine Taxonomic Services LTD. As part of this exercise, teams of divers were asked to perform observational quadrat surveys (three transects, five points along each transect, two different beds). This included (1) taking a photo, and (2) recording percent cover of eelgrass and shoot count, and (3) recording water depth from a diver’s computer. Results from each team were compared as a learning exercise. In addition to practicing field data collection, SOPs were also discussed and clarified prior to official data collection. CHRISTINE’S FIELD DETAILS.

Field replicates were included in the Workplan to represent 10% of the total number of beds. Duplicate visual surveys as well as above and belowground eelgrass collections were proposed at these points as a QA/QC measure. Due to time and tidal constraints, as well as diminished visibility after the first visual survey, only one true field replicate was taken.  High quality field data was still anticipated given the reproducibility of data from the field intercalibration as well as the comparability of the one field replicate. Several concurrent field replicate measures were proposed in the original workplan but were not taken during this Bight cycle due to logistical or funding constraints. This included infauna collection and sediment collection, which were excluded altogether from data collection, but are proposed for future Bight SAV efforts.

  - Laboratory

The primary objective of laboratory quality assurance measures was to understand both reproducibility of data and sources of error. A target of 10% of observational measurements were re-measured in the lab and labeled as laboratory replicates. Re-measurement of these replicates included: shoot count, leaves per shoot, shoot height, shoot width, and leaf area. When available, the field replicates were used as additional laboratory replicates to remeasure epiphyte biomass, aboveground, and belowground biomass. This resampling approach lays the foundation for future Data Quality Objectives for a more quantitative approach to quality assurance and control of data.

  - Data compilation and analysis

Post-data collection, we further explored the nature of errors and problems through group discussion to document improvement for future monitoring efforts. Errors and their sources were also tracked in data compilation (Table). As a general rule, most discrepancies were fixable unless they involved missing data not collected either in the field or in the lab after a sample had been already dried. Future data collections will be done through a data portal, which will circumvent most of these issues through a series of quality checks prior to submission.

Lessons learned were also tracked using an online feedback form and discussed with the Bight SAV Committee (Table) . These lessons will be implemented into the proposed Workplan for future Bight cycles and other SAV project.

Table. QA/QC summary based on submitted data. Exercise in gathering information about the nature of errors that arise in a community of practice.

| Category | Discrepancy | Counts | Fixable |
| --- | --- | --- | --- |
| Unclear SOP | Mislabeled site | 1 | 1 |
|  | Data not collected | 7 | 0 |
|  | Duplicate (field) | 1 | Not needed |
|  | Reason for overdraw not documented | 3 | 3 |
|  | Overdraw replacement not documented | 2 | 2 |
| Misinterpretation/translation error | Duplicate | 3 | 3 |
|  | Mislabeled data point | 15 | 15 |
| Missing item | Missing datapoint | 4 | 0 |
|  | Missing date | 3 | 3 |
|  | Missing site label | 10 | 10 |

Table. Lessons learned from field based on Bight ‘23 SAV Committee feedback.

| Issue | Category | Potential solution | Analysis needed to move forward |
| --- | --- | --- | --- |
| Cross3parallel method too time-consuming | Modification requested | Rely on cross-parallel method | Within-bed variability study comparing both methods (completed) |
| Two diver passes per quadrat result in marked loss of visibility for second diver | Modification requested | One pass only | Establish DQO to define and control error rates between divers |
| Potential team-to-team variability | Identification of potential issue | Build in more preset quadrat stations for future intercalibration exercises | Same as solution |
| Meadow protocol eliminates edge effects | Identification of potential issue | Consider mapping a standard area for all bed types | Comparison between standard area data (future) and non-standard area (current data) |

Results

Figure. Percent area estimates (with 95% confidence intervals) of overall eelgrass condition score by stratum. The points represent area estimates and the error bars represent the strata-based confidence intervals.

Figure. Geographic distribution of overall eelgrass condition throughout the Southern California Bight.

Figure. Percent area estimates (with 95% confidence intervals) of the seven ecosystem function scores in each of the four condition categories. Points represent area estimates, and error bars represent strata-based confidence intervals.

Figure. Percent area estimates (with 95% confidence intervals) of each of the seven ecosystem function scores in each of the four condition categories. Points represent area estimates and error bars represent strata-based confidence intervals.

Figure. Non-metric multidimensional scaling plot of ecosystem function scores at each waterbody. Points represent centroids from multiple sites at each waterbody. Points and ellipses are both color-coded by waterbody. Ellipses calculated based on waterbody. Stress = 0.041.

Figure. Total Suspended Solids (TSS) in milligrams with water quality ecosystem function scores. TSS data was not available for all sites. A linear regression (y = -0.023x + 3.3) shows a slight decrease in water quality function score with increasing TSS is observed, though not statistically significant (p=0.072). Geom_smooth() represents the 95% confidence interval for this observed relationship.

Figure. Overall eelgrass index scores with distance to inlet (km).

- Discussion

- Conclusions

- Recommendation

- Literature Cites

Appendix A – Eelgrass structural metrics

Figure. Eelgrass structural metric data by stratum.

Figure. Eelgrass structural metric data by stratum.

Appendix B – Eelgrass index and function scores

Table. Site by site ecosystem function scores and overall eelgrass condition scores.

Table. Overall eelgrass condition scores by waterbody

| waterbody | mean | sd | min | max | range | Overall condition |
| --- | --- | --- | --- | --- | --- | --- |
| Agua Hedionda Lagoon | 3.082 | 0.147 | 2.94 | 3.22 | 0.28 | good condition |
| Alamitos Bay | 1.731 | 0.412 | 1.21 | 2.32 | 1.11 | poor condition |
| Batiquitos Lagoon | 2.989 | 0.35 | 2.45 | 3.32 | 0.87 | good condition |
| Bolsa Chica | 1.917 | 0 | 1.92 | 1.92 | 0 | moderate condition |
| Dana Point Harbor | 2.243 | 0 | 2.24 | 2.24 | 0 | moderate condition |
| Huntington - Magnolia Marsh | 2.262 | 0 | 2.26 | 2.26 | 0 | moderate condition |
| King Harbor | 2.155 | 0 | 2.15 | 2.15 | 0 | moderate condition |
| La Jolla Cove | 2.598 | 0 | 2.6 | 2.6 | 0 | good condition |
| Magnolia Marsh | 2.904 | 0.666 | 2.26 | 3.55 | 1.29 | good condition |
| Mission Bay | 2.96 | 0.51 | 1.98 | 3.6 | 1.62 | good condition |
| Newport Bay | 1.944 | 0.266 | 1.52 | 2.23 | 0.71 | moderate condition |
| Oceanside Harbor | 2.91 | 0 | 2.91 | 2.91 | 0 | good condition |
| Port of Los Angeles | 2.395 | 0.78 | 1.15 | 3.1 | 1.95 | moderate condition |
| San Diego Bay | 2.462 | 0.48 | 1.64 | 3.07 | 1.43 | moderate condition |
| San Diego Bay (Zuniga Jetty) | 3.124 | 0 | 3.12 | 3.12 | 0 | good condition |
| San Diego Estuary | 1.788 | 0 | 1.79 | 1.79 | 0 | moderate condition |
| San Diego South Bay | 2.138 | 0.107 | 2.01 | 2.26 | 0.25 | moderate condition |
| San Dieguito Lagoon | 3.208 | 0 | 3.21 | 3.21 | 0 | good condition |
| San Elijo Lagoon | 1.143 | 0 | 1.14 | 1.14 | 0 | poor condition |
| Santa Ana River | 1.605 | 0 | 1.6 | 1.6 | 0 | poor condition |
| Santa Cruz Island | 2.625 | 0 | 2.62 | 2.62 | 0 | good condition |
| Seal Beach NWR | 2.477 | 0.414 | 2.08 | 2.88 | 0.8 | moderate condition |
| Seal Beach NWR (Case Pond) | 1.664 | 0 | 1.66 | 1.66 | 0 | poor condition |

Appendix C – Maps of eelgrass condition

Appendix D – Exploratory Analyses

Figure. Two-dimensional non-metric multidimensional scaling (nMDS) ordination of ecosystem function scores at each site. Ellipses calculated based on waterbody. Stress = 0.041.

Figure. Log10 – transformed chlorophyll a concentrations with overall eelgrass index scores. Chlorophyll data was not available for all sites. No significant relationship was observed.